# InControl for S7-r 
user guide

ver. July 17, 2018

| HOME SCREEN |  |
| --- | --- |
| - Lights on/off | Blue indicates ON |
| - Harvest/Travel | Tree indicates Harvest Mode, Road indicates Travel Mode; Travel mode uses auto flashers; use dial to achieve 2400 RPM |
| - Parking brake | Red indicates ON; press & hold for 5 seconds for Auto Brake (green) |
| - Sweeps up/down | Up position indicates OFF |
| - Cab pressurizer |  |
| - Engine & Bodas service |  |
| - Page Two/Return Home |  |
| - Traction |  |
| - Engine RPM control |  |
| - Flashers on/off | while in Harvest mode; always ON in Travel mode |
| - Wheel | Clockwise/counter-clockwise, push in to select |
| - Previous Screen |  |
| - Return to Home | From any screen |

NAVIGATION OPTIONS

- Touch Screen

- Soft Keys

- Wheel (13)

- (14) Previous Screen & (15) Home

FOR YOUR SAFETY

- While in E-Shake or Custom mode, unclamp or Joystick trigger will stop & cycle

- Only make adjustments while machine is in Park

| ADJUSTING RPM |  |
| --- | --- |
|  | Press 10 to access engine RPM |
|  | Use Wheel (13) to raise/lower RPM; clockwise increases, counter-clockwise decreases |
|  | - Press Rabbit for Harvest RPM, press Turtle to return to idle - Rabbit default is 2200 RPM - Turtle default is Idle |
|  | Below 2000 RPM, shake will not engage, indicated by yellow RPM icon Above 2000 RPM, shake will engage, indicated by green RPM icon |
|  | In travel/road, use dial to achieve 2400 RPM |

| AXIOM FEATURE |  |
| --- | --- |
|  | AXIOM is an optional feature that will be indicated on the Home screen |
|  | - Use to Raise - Use to Lower - Once desired height is reached press above icon to return to Home screen & enable throttle - While AXIOM is elevated, Travel mode is disabled. |
| SERVICE DIAGNOSTICS |  |
|  | Flashes red when service is required; pressing will display engine & equipment fault codes |
|  | Indicates no water in the tank OR malfunction in monitoring pressure switch |
|  | Press 14 & 15 simultaneously to access Quick Diagnostics Page |
|  | Press & HOLD Pad Water icon to prime water pump |

| SETTING PASSWORD |  |
| --- | --- |
|  | Most functions on Page 2 are password protected. If the icon is red, no changes can be made. To input password, press Password icon on any screen |
|  | A blue Password screen will pop up |
|  | - Using Wheel (13), select number by scrolling then push in to enter the number |
|  | - Or touch 0-0-0-0 & key pad will display; input desired code & press OK |
|  | If desired code is entered password icon becomes green |
|  | Factory password is 1944 |
|  | TO CHANGE PASSWORD - 1944 will always work as default - Follow above steps to enter correct password - Once icon is green press Password again; at blue Password pop-up press Reset - Use above steps to enter new desired password - Now new and Factory password will access system |

| PAGE TWO |  |
| --- | --- |
|  | From the Home screen, touch the Arrow to move forward to Page Two |
|  | Once in Page Two, touch desired Page icon or rotate wheel & depress when desired Page is highlighted yellow |
| HOME |  |
|  | Press to return to Home OR press (14) or (15) |

| SHAKE CONTROL |  |
| --- | --- |
|  | - Shake Control Screen varies by Option selected - Shake Control can be accessed while in E-Shake through the Joystick trigger. This is useful when shaking a smaller replant. |
|  | Option: Manual HD – MD – LD |
|  | - Shows displacement range for HD/MD/LD - Adjustable in range- push Adjust - Once highlighted move w/ finger or corresponding button - Mode allows operator to use displacement pedal to vary shake within range |
|  | When in Manual Mode, Home screen will display like this |
|  | Option: E-Shake |
|  | - Amplitude = amount of shake - Period = length of shake - Number of Cycles = number of times repeats cycle - Peak Hold = amount of time at peak amplitude After adjustment to above options, press & HOLD clamp/shake button to visually graph sequence |
|  | When in E-Shake mode, Home screen display like this; when in E-Shake mode, auto-clamp feature is active |
|  | Option: Custom Shake 1-4 |
|  | - Use wheel to move & select position or time; depress & rotate wheel to adjust. - Position is amount shake - As shake sequence is produced, graph will visually indicate pattern. |
|  | When in Custom Mode, Home screen will display like this; when in Custom Mode, auto-clamp feature is active Correlates to custom cycle chosen |
| REAR CAMERA |  |
|  | - Allows for larger viewing - Traveling in reverse also displays full screen camera |

| TREE COUNT (JOB COUNT) |  |
| --- | --- |
|  | - Job count can be reset by pushing Job Count Reset button - Clamp/shake & forward travel is needed to count as a job |

| SHAKE THRESHOLD (CLAMP PRESSURE ADJUSTMENT) |  |
| --- | --- |
|  | - Press Set Up |
|  | - Desired clamp pressure changes to teal |
|  | - Press & HOLD Clamp button & rotate (13) wheel. Clockwise increases clamp pressure, counter-clockwise decreases clamp pressure. Once desired pressure is met release Clamp button on joystick. |
|  | Current clamp pressure will be displayed on Home screen |
|  | - Also in this screen you can turn water off/on for testing & cleanliness reasons if in a shop & testing the head - Press this icon to turn the pad water on/off - Press this icon to turn the sling water on/off |
|  | - Free Wheel Mode - Free Wheel Mode is the preferred mode. - It allows the machine to position itself correctly when shaker-head-to-tree alignment is improper. - Free Wheel Mode should not be used in hilly terrain. - Free Wheel Mode enabled. |
|  | - Press this icon to disabled Free Wheel Mode. |

| OPERATOR ADJUSTMENT |  |
| --- | --- |
|  | - Sweeper On Time = time brush will rotate after stop - Sweeper = adjust speed of sweeper rotation - Acceleration = Increase/decrease speed of shaker acceleration - Deceleration = increase/decrease speed of shaker deceleration Highlighted icon turns yellow, then drag finger OR use wheel (13) to increase/decrease |
|  | - On longer spacing shaker will begin to shift for quicker tree-to-tree times. If spacing is below 18’, 1st Gear Only may be used to avoid shifting. |
|  | - 2nd Gear Only will put shaker’s front wheel in minimum displacement which will keep front tire from spinning in loose soil conditions or when loading equipment on trailers. |
|  | - Pad Lube = increase/decrease water to filled pads - Sling Lube = increase/decrease sling lube; typical range is .5-1.0 sec of water per injection cycle. - Tree Release % = amount head will open in auto shake mode - Clamp Open = increase/decrease speed of clamp open - Clamp Close = increase/decrease speed of clamp close Highlighted icon turns yellow, then drag finger OR use wheel (13) to increase/decrease |
|  | - Head Retract = increase/decrease speed of head retract - Head Extend = increase/decrease speed of head extend - Head Raise = increase/decrease speed of head raise - Head Lower = increase/decrease speed of head lower Highlighted icon turns yellow, then drag finger OR use wheel (13) to increase/decrease |

| BODAS SERVICE |  |
| --- | --- |
|  | Allows access for factory/dealer service on equipment system |